FALL 2024

RCA 2020

I2C drivers for the RCA 2020

Implementation and Test Document

RCA2020

An RCA CDP1802 COSMAC revival project

Designed by Richard van Harderwijk

I2C drivers Implementation and Test Document

| Version | Date | Author | Description/Changes |
| --- | --- | --- | --- |
| 1.0 | Nov 2024 | Richard van Harderwijk | First Version |

Content

Introduction	4

Implementation	5

A brief description of the solution	5

Implementation and driver calling	5

Testing	6

Use of the PCF8563 (real time clock/calendar) as testdevice	6

Testresults	6

I2C bus analysis	6

Conclusions	7

Annex A – References and Additional Documentation	8

Annex B – Listings	9

Controller Transmitter mode	9

Controller Receiver mode	9

Initialization	9

Test Program	14

# Introduction

About this document

This document describes the implementation and testing of the drivers for the RCA2020 I2C interface [1].

The RCA2020 system is an SBC (Single Board Computer) based on the 1970’s RCA CDP1802 COSMAC CPU. While specific for this system, the interface should be easily portable to any other 1802 system, both the hardware and the software.

Terminology

The industry moved away from the old terms master and slave to controller and peripheral. These terms are used in this document, but in references you may still find the older terms.

# Implementation

## A brief description of the solution

A part of the RCA2020 system [1] is an interface from the 1802 CPU to the I2C bus.

The RCA2020 I2C system consists of two hardware parts:

- The core: a controller chip (PCA9564) [3], [4] connected to the CPU, as described in the design, build and test manual [1]. This interfaces the CPU to the I2C bus.

- An on-board peripheral: a date/time and alarm chip (PCF8563) [1] connected via the I2C bus.

The four registers of the controller chip are mapped to INP/OUT 4..7 (not memory mapped).

The I2C interface design criteria are described in the design document [6]

## Implementation and driver calling

Example flowcharts are excellently described in the application note of the bus controller (AN10148) [4] [6]. Three drivers are implemented:

- I2C Controller Transmitter mode (driver code start at $7000)

- I2C Controller Receiver mode (driver code start at $7100)

- I2C Controller Initialization Sequence (driver code start at $7200)

The drivers are page relocatable.

The listings are in the annexes and on github [6] including the binaries, etc.

The routines expect a defined stack R(X) for the INP/OUT commands.The use of register R(7) and R(8) for calling and return codes are described in the design document, and in the listings. The routines return with SEP 5 (SCRT return).

As a side note: asking ChatGPT to create these drivers was a hallucinating experience with mnemonics and op-codes from all CPU’s except the 1802… We’re still not obsolete...

# Testing

## Use of the PCF8563 (real time clock/calendar) as testdevice

The onboard RTC chip is used as testdevice. The testprogram initializes the RTC [7], [8] just before new years eve and reads the date/time after a delay. Initialization is done by sending a 17 byte string, denoting the startregister to write (0x00) and then 16 bytes for the 16 registers of the PCF8563.

## Testresults

Find below the run and results of the testprogram. The RCA2020 uses UT4 [9], [1] as operating system-ish. The I2C routines return values are: 0x0000 indicating no errors.

......*$P

......

......Start I2C test PCF8563 Real time clock

......TX return value: 0000

......Delay

......RX return value: 0000

......End test. Press return.

......

......*?M F0 30

......00F0 6E2E 0D0A 0000 0000 5859 2331 0012 2380;

......0100 8080 8000 03FF 0000 0400 0001 0101 2480;

......0110 8080 8004 07FF 0000 22C2 00E0 4484 4100

......*

Happy New year!

The yellow marked string indicates the date/time set =>

23:59:58 on Sunday Dec 31, 2023

The green marked string indicates the date/time read after the delay=>

00:00:04 on Monday Jan 1, 2024

## I2C bus analysis

A screengrab of the connected logic analyzer while transmitting is seen below. It shows the sending of the peripheral address (0x51); 0x00 to indicate start writing at PCF8563 register 0; 0x00 for regs 0, 1; 0x58 for the seconds (reg 2) and 0x59 for the minutes (reg 3).

It also indicates that the 1802 and the routines are not fast enough to saturate the I2C channel.

## Conclusions

- The drivers work.

- The hardware (I2C part of the RCA2020) works.

- The hardware I2C controller (PCA9564) and drivers give the 1802 a robust I2C solution, easily implementable on other 1802 systems (‘only’ needing INP/OUT 4..7).

- The RTC chip works, giving the RCA2020 a real-time clock, calendar, and timers (with interrupt capability).

- The implementation of the drivers (with all error trapping) makes the drivers too slow to saturate the I2C channel. This is not a big problem since the interface is not meant for block transfer of large amounts of data.

# Annex A – References and Additional Documentation

References

[1]   RCA2020 system/github

https://github.com/richardvanharderwijk/1802-SBC-the-RCA2020

[2]   MPM-201 User Manual for the CDP1802 COSMAC Microprocessor http://www.bitsavers.org/pdf/rca/1802/MPM-201A_CDP1802_User_Manual_1976.pdf

[3]   Data sheet PCA9564 (I2C-bus controller)

https://www.nxp.com/docs/en/data-sheet/PCA9564.pdf

[4]   AN10148: PCA9564 – I2C-bus controller https://www.nxp.com/docs/en/application-note/AN10148.pdf

[5]   The Book Of I2C – Randall Hyde; ISBN 9781718502468

[6]   RCA2020 I2C/github (the design document, listing, code, etc)

https://github.com/richardvanharderwijk/1802-I2C-interface-for-RCA2020

[7]   Data sheet PCF8563 (date/time and alarm chip)

https://www.nxp.com/docs/en/data-sheet/PCF8563.pdf

[8]   UM10301: User Manual for NXP Real Time Clocks PCF8563, …

https://www.nxp.com/docs/en/user-guide/UM10301.pdf

[9]   EVALUATION KIT MANUAL FOR THE RCA CDP1802 COSMAC MICROPROCESSOR MPM-203

http://www.bitsavers.org/components/rca/cosmac/MPM-203_CDP1802_Evaluation_Kit_Manual_Sep76.pdf

Additional Documentation

Libraries for using various I2C devices with an 1802-Mini with the PIO and I2C expansion boards. – Various contributors from the COSMAC ELF Group cosmacelf@groups.io - https://github.com/fourstix/Elfos-I2C-Libraries/tree/main?tab=readme-ov-file

# Annex B – Listings

## Controller Transmitter mode

## Controller Receiver mode

## Initialization

0000                             ;

0000                             ;

0000                             ;	Driver: I2C Controller Transmitter mode

0000                             ;	For 1802 CPU - RCA 2020 SBC with PCA9564 Controller

0000                             ;	Version: 0.1

0000                             ;	Date: Oct 31, 2024

0000                             ;	Author: Richard van Harderwijk

0000                             ;

0000                             ;	PCA9564 connected via INP/OUT 4..7

0000                             ;	Doc: https://github.com/richardvanharderwijk/1802-I2C-interface-for-RCA2020

0000                             ;	Doc: AN10148 application note, figure 6a

0000                             ;

0000                             ;	Pre: R(8) is pointer to string of bytes to send; R(7).1 is peripheral address; R(7).0 is #bytes to send

0000                             ;	Post on success: R(7).1 and R(7).0 contain 0x00

0000                             ;	Post on error: R(7).1 is Expected I2CSTA value; R(7).0 is Actual I2CSTA returned value

0000                             ;	End with SCRT return D5

0000                             ;

0000                             ;

7000                          .ORG   $7000

7000                BEGIN_TX:

7000   87                     GLO   7   	; test on #bytes to send > 0

7001   3A 09                  BNZ   start_tx

7003   F8 01                  LDI   0x01

7005   A7                     PLO   7

7006   B7                     PHI   7

7007   30 74                  BR   end_tx   	; exit w/ error

7009                START_TX:

7009   F8 E4                  LDI   0xE4   	; generate I2C start command

700B   73                     STXD

700C   73                     STXD

700D   60                     IRX

700E   67                     OUT   7   	; I2CCON register

700F                CHK_SI_1:      		; wait for SI = 1

700F   6F                     INP   7

7010   FA 08                  ANI   0x08

7012   32 0F                  BZ   chk_SI_1

7014   6C                     INP   4   	; I2CSTA register, check

7015   FB 08                  XRI   0x08

7017   32 20                  BZ   continue1 ; ok, continue

7019   F0                     LDX

701A   A7                     PLO   7

701B   F8 08                  LDI   0x08

701D   B7                     PHI   7

701E   30 74                  BR   end_tx   	; exit w/ error

7020                CONTINUE1:

7020   97                     GHI   7   	; peripheral address to I2CDAT

7021   FE                     SHL      	; Shift Left, address in MSB bits [7..1]

7022   FA FE                  ANI   0xFE   	; R/W bit = 0 [LSB]

7024   73                     STXD

7025   73                     STXD

7026   60                     IRX

7027   65                     OUT   5   	; I2CDAT register

7028   F8 C4                  LDI   0xC4   	; I2C send peripheral address + write command

702A   73                     STXD

702B   73                     STXD

702C   60                     IRX

702D   67                     OUT   7

702E                CHK_SI_2:      		; wait for SI=1

702E   6F                     INP   7

702F   FA 08                  ANI   0x08

7031   32 2E                  BZ   chk_SI_2

7033   6C                     INP   4   	; I2CSTA register, check

7034   FB 18                  XRI   0x18

7036   32 3F                  BZ   continue2

7038   F0                     LDX

7039   A7                     PLO   7

703A   F8 18                  LDI   0x18

703C   B7                     PHI   7

703D   30 74                  BR   end_tx   	; exit w/ error

703F                CONTINUE2:      		; loop for byte string to send

703F   48                     LDA   8   	; load data to I2CDAT

7040   73                     STXD

7041   73                     STXD

7042   60                     IRX

7043   65                     OUT   5

7044   F8 C4                  LDI   0xC4   	; send data command

7046   73                     STXD

7047   73                     STXD

7048   60                     IRX

7049   67                     OUT   7

704A                CHK_SI_3:      		; wait for SI = 1, poll for transmission finished

704A   6F                     INP   7

704B   FA 08                  ANI   0x08

704D   32 4A                  BZ   chk_SI_3

704F   6C                     INP   4   	; I2CSTA register, check

7050   FB 28                  XRI   0x28

7052   32 5B                  BZ   continue3

7054   F0                     LDX

7055   A7                     PLO   7

7056   F8 28                  LDI   0x28

7058   B7                     PHI   7

7059   30 74                  BR   end_tx   	; exit w/ error

705B                CONTINUE3:

705B   27                     DEC   7   	; more bytes to send?

705C   87                     GLO   7

705D   3A 3F                  BNZ   continue2

705F   F8 D4                  LDI   0xD4   	; generate stop command

7061   73                     STXD

7062   73                     STXD

7063   60                     IRX

7064   67                     OUT   7

7065   6F                     INP   7   	; I2CCON register, check STO bit = 0

7066   FA 10                  ANI   0x10

7068   32 70                  BZ   continue4

706A   F8 02                  LDI   0x02

706C   A7                     PLO   7

706D   B7                     PHI   7

706E   30 74                  BR   end_tx   	; exit w/ error

7070                CONTINUE4:

7070   F8 00                  LDI   0x00   	; exit w/ success :-)

7072   A7                     PLO   7

7073   B7                     PHI   7

7074                END_TX:

7074   D5                     SEP   5

7075                             ;

7075                             ;

7075                             ;	Driver: I2C Controller Receiver mode

7075                             ;	For 1802 CPU - RCA 2020 SBC with PCA9564 Controller

7075                             ;	Version: 0.1

7075                             ;	Date: Oct 31, 2024

7075                             ;	Author: Richard van Harderwijk

7075                             ;

7075                             ;	PCA9564 connected via INP/OUT 4..7

7075                             ;	Doc: https://github.com/richardvanharderwijk/1802-I2C-interface-for-RCA2020

7075                             ;	Doc: AN10148 application note, figure 7a

7075                             ;

7075                             ;	Pre: R(8) is pointer to memory allocated to store bytes received;

7075                             ;	Pre: R(7).1 is peripheral address; R(7).0 is #bytes to receive

7075                             ;	Post on success: R(7).1 and R(7).0 contain 0x00

7075                             ;	Post on error: R(7).1 is Expected I2CSTA value; R(7).0 is Actual I2CSTA returned value

7075                             ;	End with SCRT return D5

7075                             ;

7075                             ;

7100                          .ORG   $7100

7100                BEGIN_RX:

7100   87                     GLO   7   	; test on #bytes to receive > 0

7101   3A 09                  BNZ   start_rx

7103   F8 01                  LDI   0x01

7105   A7                     PLO   7

7106   B7                     PHI   7

7107   30 90                  BR   end_rx   	; exit w/ error

7109                START_RX:

7109   F8 E4                  LDI   0xE4   	; generate I2C start command

710B   73                     STXD

710C   73                     STXD

710D   60                     IRX

710E   67                     OUT   7   	; I2CCON register

710F                CHK_SI_4:      		; wait for SI = 1

710F   6F                     INP   7

7110   FA 08                  ANI   0x08

7112   32 0F                  BZ   chk_SI_4

7114   6C                     INP   4   	; I2CSTA register, check

7115   FB 08                  XRI   0x08

7117   32 20                  BZ   continue5 ; ok, continue

7119   F0                     LDX

711A   A7                     PLO   7

711B   F8 08                  LDI   0x08

711D   B7                     PHI   7

711E   30 90                  BR   end_rx   	; exit w/ error

7120                CONTINUE5:

7120   97                     GHI   7   	; peripheral address to I2CDAT

7121   FE                     SHL      	; Shift Left, address in MSB bits [7..1]

7122   F9 01                  ORI   0x01   	; R/W bit = 1 [LSB]

7124   73                     STXD

7125   73                     STXD

7126   60                     IRX

7127   65                     OUT   5   	; I2CDAT register

7128   F8 C4                  LDI   0xC4   	; I2C send peripheral address + write command

712A   73                     STXD

712B   73                     STXD

712C   60                     IRX

712D   67                     OUT   7

712E                CHK_SI_5:      		; wait for SI=1

712E   6F                     INP   7

712F   FA 08                  ANI   0x08

7131   32 2E                  BZ   chk_SI_5

7133   6C                     INP   4   	; I2CSTA register, check

7134   FB 40                  XRI   0x40

7136   32 3F                  BZ   continue6

7138   F0                     LDX

7139   A7                     PLO   7

713A   F8 40                  LDI   0x40

713C   B7                     PHI   7

713D   30 90                  BR   end_rx   	; exit w/ error

713F                CONTINUE6:

713F   27                     DEC   7   	; last byte of receive is not acknowledged

7140   87                     GLO   7   	; if only one byte to read, skip read loop

7141   32 61                  BZ   read_last_byte

7143                CONTINUE7:      		; loop read all but last byte

7143   F8 C4                  LDI   0xC4   	; read data command

7145   73                     STXD

7146   73                     STXD

7147   60                     IRX

7148   67                     OUT   7

7149                CHK_SI_6:      		; wait for SI = 1, poll for transmission finished

7149   6F                     INP   7

714A   FA 08                  ANI   0x08

714C   32 49                  BZ   chk_SI_6

714E   6C                     INP   4  	; I2CSTA register, check

714F   FB 50                  XRI   0x50

7151   32 5A                  BZ   continue8

7153   F0                     LDX

7154   A7                     PLO   7

7155   F8 50                  LDI   0x50

7157   B7                     PHI   7

7158   30 90                  BR   end_rx   	; exit w/ error

715A                CONTINUE8:

715A   6D                     INP   5   	; read data from I2C bus from I2CDAT

715B   58                     STR   8   	; store received I2C data via R(8) pointer

715C   18                     INC   8

715D   27                     DEC   7   	; more bytes to receive?

715E   87                     GLO   7

715F   3A 43                  BNZ   continue7

7161                READ_LAST_BYTE:

7161   F8 44                  LDI   0x44   	; read data command

7163   73                     STXD      	; to read last byte, not acknowledged

7164   73                     STXD

7165   60                     IRX

7166   67                     OUT   7

7167                CHK_SI_7:      		; wait for SI = 1, poll for transmission finished

7167   6F                     INP   7

7168   FA 08                  ANI   0x08

716A   32 67                  BZ   chk_SI_7

716C   6C                     INP   4   	; I2CSTA register, check

716D   FB 58                  XRI   0x58

716F   32 78                  BZ   continue9

7171   F0                     LDX

7172   A7                     PLO   7

7173   F8 58                  LDI   0x58

7175   B7                     PHI   7

7176   30 90                  BR   end_rx   	; exit w/ error

7178                CONTINUE9:

7178   6D                     INP   5   	; read last byte from I2C bus from I2CDAT

7179   58                     STR   8   	; store received I2C data via R(8) pointer

717A   18                     INC   8

717B   F8 D4                  LDI   0xD4   	; generate stop command

717D   73                     STXD

717E   73                     STXD

717F   60                     IRX

7180   67                     OUT   7

7181   6F                     INP   7   	; I2CCON register, check STO bit

7182   FA 10                  ANI   0x10

7184   32 8C                  BZ   continue10

7186   F8 02                  LDI   0x02

7188   A7                     PLO   7

7189   B7                     PHI   7

718A   30 90                  BR   end_rx   	; exit w/ error

718C                CONTINUE10:

718C   F8 00                  LDI   0x00   	; exit w/ success :-)

718E   A7                     PLO   7

718F   B7                     PHI   7

7190                END_RX:

7190   D5                     SEP   5

7191                             ;

7191                             ;

7191                             ;	Driver: I2C Controller Initialization Sequence

7191                             ;	For 1802 CPU - RCA 2020 SBC with PCA9564 Controller

7191                             ;	Version: 0.1

7191                             ;	Date: Oct 31, 2024

7191                             ;	Author: Richard van Harderwijk

7191                             ;

7191                             ;	PCA9564 connected via INP/OUT 4..7

7191                             ;	Doc: https://github.com/richardvanharderwijk/1802-I2C-interface-for-RCA2020

7191                             ;	Doc: AN10148 application note, figure 5

7191                             ;

7191                             ;	Pre: R(7).1 future use (own peripheral address); R(7).0 future use (clock frequency)

7191                             ;	In this version clock frequency is set at 88 kHz

7191                             ;	Post on success: R(7).1 and R(7).0 contain 0x00

7191                             ;	Post on error: future use, in this version no test

7191                             ;	End with SCRT return D5

7191                             ;

7191                             ;

7200                          .ORG   $7200

7200                BEGIN_INIT:

7200   F8 FF                  LDI   0xFF   	; 0xFF in TimeOut register

7202   73                     STXD

7203   73                     STXD

7204   60                     IRX

7205   64                     OUT   4   	; I2CTO register

7206   F8 64                  LDI   0x64   	; own peripheral address

7208   73                     STXD

7209   73                     STXD

720A   60                     IRX

720B   66                     OUT   6   	; I2CADR register

720C   F8 44                  LDI   0x44   	; Enable Serial IO, 88 kHz

720E   73                     STXD

720F   73                     STXD

7210   60                     IRX

7211   67                     OUT   7   	; I2CCON register

7212   F8 2F                  LDI   0x2F   	; wait for oscillator startup

7214                DELAY1:

7214   FF 01                  SMI   0x01

7216   3A 14                  BNZ   delay1

7218   F8 C4                  LDI   0xC4   	; peripheral receiver mode

721A   73                     STXD

721B   73                     STXD

721C   60                     IRX

721D   67                     OUT   7   	; I2CCON register

721E   F8 00                  LDI   0x00   	; exit always w/ success :-)

7220   A7                     PLO   7

7221   B7                     PHI   7

7222                END_INIT:

7222   D5                     SEP   5

BEGIN_TX:           7000 DEFINED AT LINE 31

START_TX:           7009 DEFINED AT LINE 40

> USED AT LINE 34

CHK_SI_1:           700F DEFINED AT LINE 47

> USED AT LINE 50

CONTINUE1:          7020 DEFINED AT LINE 61

> USED AT LINE 54

CHK_SI_2:           702E DEFINED AT LINE 75

> USED AT LINE 78

CONTINUE2:          703F DEFINED AT LINE 89

> USED AT LINE 82

> USED AT LINE 119

CHK_SI_3:           704A DEFINED AT LINE 102

> USED AT LINE 105

CONTINUE3:          705B DEFINED AT LINE 116

> USED AT LINE 109

CONTINUE4:          7070 DEFINED AT LINE 135

> USED AT LINE 129

END_TX:             7074 DEFINED AT LINE 140

> USED AT LINE 38

> USED AT LINE 59

> USED AT LINE 87

> USED AT LINE 114

> USED AT LINE 133

BEGIN_RX:           7100 DEFINED AT LINE 169

START_RX:           7109 DEFINED AT LINE 179

> USED AT LINE 172

CHK_SI_4:           710F DEFINED AT LINE 186

> USED AT LINE 189

CONTINUE5:          7120 DEFINED AT LINE 200

> USED AT LINE 193

CHK_SI_5:           712E DEFINED AT LINE 214

> USED AT LINE 217

CONTINUE6:          713F DEFINED AT LINE 228

> USED AT LINE 221

CONTINUE7:          7143 DEFINED AT LINE 233

> USED AT LINE 261

CHK_SI_6:           7149 DEFINED AT LINE 240

> USED AT LINE 243

CONTINUE8:          715A DEFINED AT LINE 254

> USED AT LINE 247

READ_LAST_BYTE:     7161 DEFINED AT LINE 263

> USED AT LINE 231

CHK_SI_7:           7167 DEFINED AT LINE 270

> USED AT LINE 273

CONTINUE9:          7178 DEFINED AT LINE 284

> USED AT LINE 277

CONTINUE10:         718C DEFINED AT LINE 303

> USED AT LINE 297

END_RX:             7190 DEFINED AT LINE 308

> USED AT LINE 176

> USED AT LINE 198

> USED AT LINE 226

> USED AT LINE 252

> USED AT LINE 282

> USED AT LINE 301

BEGIN_INIT:         7200 DEFINED AT LINE 336

DELAY1:             7214 DEFINED AT LINE 357

> USED AT LINE 359

END_INIT:           7222 DEFINED AT LINE 371

## Test Program

0000                          .ORG   $0000

0000                             ;

0000                             ;

0000                             ;	Testprogram for I2C Driver with PCF8563 RealTime Clock

0000                             ;	For 1802 CPU - RCA 2020 SBC with PCA9564 Controller

0000                             ;	Version: 0.1

0000                             ;	Date: Nov 17, 2024

0000                             ;	Author: Richard van Harderwijk

0000                             ;

0000                             ;	Uses UT4 and I2C driver routines

0000                             ;	Doc: https://github.com/richardvanharderwijk/1802-I2C-interface-for-RCA2020

0000                             ;

0000                             ;	Uses SCRT

0000                             ;

0000                             ;

0000                BEGIN_TST:

0000                             ; enter here at $0000 with X, P = 0, 0 via UT4 (int disabled)

0000                INIT:

0000   F8 7F                  LDI   0x7F   	; init stack in R2

0002   A2                     PLO   2

0003   F8 FE                  LDI   0xFE

0005   B2                     PHI   2

0006   E2                     SEX   2

0007                             		;

0007   F8 00                  LDI   0x00   	; init PC in R3

0009   B3                     PHI   3

000A   F8 0E                  LDI   0x0E

000C   A3                     PLO   3

000D   D3                     SEP   3

000E                             		;

000E   F8 C3                  LDI   0xC3   	; init SCRT call in R4, return in R5

0010   B4                     PHI   4

0011   B5                     PHI   5

0012   F8 01                  LDI   0x01

0014   A4                     PLO   4

0015   F8 11                  LDI   0x11

0017   A5                     PLO   5

0018                             		;

0018                RUN_TST:

0018   F8 00                  LDI   0x00   	; init pointer to text to print

001A   B9                     PHI   9

001B   F8 82                  LDI   0x82

001D   A9                     PLO   9

001E                             		;

001E                PRINT1:

001E   49                     LDA   9   	; get byte to print and put in R(F).1

001F   BF                     PHI   F

0020   32 27                  BZ   cont1   	; exit if byte = 0x00 (EOF)

0022   D4                     SEP   4

0023   C2 A4                  DW   0xC2A4   	; SCRT to UT4: TYPE @ 0xC2A4

0025   30 1E                  BR   print1

0027                CONT1:

0027                             		;

0027                             		; initialization routine of I2C controller

0027   D4                     SEP   4   	; SCRT to I2C controller init

0028   72 00                  DW   0x7200

002A                             		; send initialization string to 8563 RTC

002A   F8 00                  LDI   0x00   	; init pointer to bytes to send

002C   B8                     PHI   8

002D   F8 F5                  LDI   0xF5

002F   A8                     PLO   8

0030                             		;

0030   F8 51                  LDI   0x51   	; peripheral/RTC address (0xA2 write)

0032   B7                     PHI   7

0033   F8 11                  LDI   0x11   	; 17 bytes string for RTC registers

0035   A7                     PLO   7

0036                             		;

0036   D4                     SEP   4   	; SCRT to I2C transmitter routine

0037   70 00                  DW   0x7000

0039                             		;

0039   97                     GHI   7   	; print return values

003A   BF                     PHI   F

003B   D4                     SEP   4

003C   C2 AE                  DW   0xC2AE

003E   87                     GLO   7

003F   BF                     PHI   F

0040   D4                     SEP   4

0041   C2 AE                  DW   0xC2AE

0043                             		; 8563 RTC initialization ready

0043                             		;

0043   F8 00                  LDI   0x00   	; init pointer to text to print

0045   B9                     PHI   9   	; print delay message

0046   F8 BE                  LDI   0xBE

0048   A9                     PLO   9

0049                             		;

0049                PRINT2:

0049   49                     LDA   9   	; get byte to print and put in R(F).1

004A   BF                     PHI   F

004B   32 52                  BZ   cont2   	; exit if byte = 0x00 (EOF)

004D   D4                     SEP   4

004E   C2 A4                  DW   0xC2A4   	; SCRT to UT4: TYPE @ 0xC2A4

0050   30 49                  BR   print2

0052                CONT2:

0052                             		;

0052   F8 FF                  LDI   0xFF   	; delay

0054   B9                     PHI   9

0055                DELAY2:

0055   29                     DEC   9

0056   C4                     NOP

0057   C4                     NOP

0058   C4                     NOP

0059   C4                     NOP

005A   99                     GHI   9

005B   3A 55                  BNZ   delay2

005D                             		;

005D   F8 51                  LDI   0x51   	; peripheral/RTC address (0xA3 read)

005F   B7                     PHI   7

0060   F8 11                  LDI   0x11   	; 17 bytes string for RTC registers to read

0062   A7                     PLO   7

0063                             		;

0063                             		; R(8) already points after init (sent) string for read

0063   D4                     SEP   4   	; SCRT to I2C receiver routine

0064   71 00                  DW   0x7100

0066                             		;

0066   97                     GHI   7   	; print return values

0067   BF                     PHI   F

0068   D4                     SEP   4

0069   C2 AE                  DW   0xC2AE

006B   87                     GLO   7

006C   BF                     PHI   F

006D   D4                     SEP   4

006E   C2 AE                  DW   0xC2AE

0070                             		; 8563 RTC read registers after delay ready

0070                             		;

0070   F8 00                  LDI   0x00   	; init pointer to text to print

0072   B9                     PHI   9

0073   F8 D9                  LDI   0xD9

0075   A9                     PLO   9

0076                             		;

0076                PRINT3:

0076   49                     LDA   9   	; get byte to print and put in R(F).1

0077   BF                     PHI   F

0078   32 7F                  BZ   cont3   	; exit if byte = 0x00 (EOF)

007A   D4                     SEP   4

007B   C2 A4                  DW   0xC2A4   	; SCRT to UT4: TYPE @ 0xC2A4

007D   30 76                  BR   print3

007F                CONT3:

007F                             		;

007F                END_TST:

007F   C0 C1 00               LBR   0xC100   ; Jump to UT4 to examine return string in memory

0082                             		;

0082                             		;

0082                BEGIN_TXT:

0082   0D 0A 53 74 61 72 74 20 49 32 43 20 74 65 73 74 20 50 43 46 38 35 36 33 20 52 65 61 6C 20 74 69 6D 65 20 63 6C 6F 63 6B 0D 0A 54 58 20 72 65 74 75 72 6E 20 76 61 6C 75 65 3A 20 00 DB   0x0D,0x0A,"Start I2C test PCF8563 Real time clock",0x0D,0x0A,"TX return value: ",0x00

00BE                DELAY_TXT:

00BE   0D 0A 44 65 6C 61 79 0D 0A 52 58 20 72 65 74 75 72 6E 20 76 61 6C 75 65 3A 20 00 DB   0x0D,0x0A,"Delay",0x0D,0x0A,"RX return value: ",0x00

00D9                END_TXT:

00D9   0D 0A 45 6E 64 20 74 65 73 74 2E 20 50 72 65 73 73 20 72 65 74 75 72 6E 2E 0D 0A 00 DB   0x0D,0x0A,"End test. Press return.",0x0D,0x0A,0x00

00F5                INIT_CLOCK_STRING:

00F5   00 00 00 58 59 23 31 00 12 23 80 80 80 80 00 03 FF DB   0x00,0x00,0x00,0x58,0x59,0x23,0x31,0x00,0x12,0x23,0x80,0x80,0x80,0x80,0x00,0x03,0xFF

BEGIN_TST:          0000 DEFINED AT LINE 20

INIT:               0000 DEFINED AT LINE 24

RUN_TST:            0018 DEFINED AT LINE 46

PRINT1:             001E DEFINED AT LINE 52

> USED AT LINE 58

CONT1:              0027 DEFINED AT LINE 59

> USED AT LINE 55

PRINT2:             0049 DEFINED AT LINE 95

> USED AT LINE 101

CONT2:              0052 DEFINED AT LINE 102

> USED AT LINE 98

DELAY2:             0055 DEFINED AT LINE 106

> USED AT LINE 113

PRINT3:             0076 DEFINED AT LINE 140

> USED AT LINE 146

CONT3:              007F DEFINED AT LINE 147

> USED AT LINE 143

END_TST:            007F DEFINED AT LINE 151

BEGIN_TXT:          0082 DEFINED AT LINE 157

DELAY_TXT:          00BE DEFINED AT LINE 160

END_TXT:            00D9 DEFINED AT LINE 163

INIT_CLOCK_STRING:  00F5 DEFINED AT LINE 166